ADONIS static white, tunable white, RGB and RGBW

ADONIS: 38,6 mm × 28 mm (W × H)
Mounting types: surface, wall, ground mounting, adjustable

Fully encapsulated in polyurethane with a clear polyurethane top layer, providing high-quality waterproof sealing and UV resistance. The polyurethane formulation has been specifically developed for LED technology. Resistant to corrosive gases such as sulfur, chlorine, or salt mist. Fully automated and reliable in-house encapsulation process with 100% functional testing and outgoing goods inspection of each luminaire.

Extruded and powder-coated aluminum housing (EN AW-6060 T66) in black (RAL 9004), white (RAL 9003), and grey (RAL 9022). With a coating thickness of 50–100 µm for optimal color quality and high surface durability. Custom colors available on request. Equipped with an IP67 cable and connector system. Mounting accessories included.

High-quality linear optics with three different light distribution options (10°, 30°, and 60°), and fully dot-free, homogeneous opal encapsulation without color shift across the beam angle (less than +/- 50 K measured over the entire hemisphere).

Mounting accessory

- Surface mounting, horizontally or vertically fixed.

- Wall mounting with adjustable arm in four different offsets made of powder-coated aluminum with integrated cable channel: 40 mm, 100 mm, 200 mm, and 300 mm wall distance; custom lengths available on request. Luminaire can be continuously adjusted by +/- 120°.

- In-ground installation channel (aluminum, silver anodized) with drainage holes and sufficient space for cable management. Available either as a single unit cut precisely to luminaire length or as a continuous line installation with mechanical connectors. External IK10 certification when installed in the in-ground channel.

- Surface mounting with screw fixing for additional protection against vandalism, theft, or extreme weather conditions.

All accessories are available in three color options: RAL 9004 (black), RAL 9003 (white), and RAL 9022 (grey). Custom colors available on request. Surface clips are concealed beneath the luminaire and are not visible externally.

Additional glare control accessory

- Aluminum glare shield profile, powder-coated in luminaire colors (custom colors available on request), for one-sided light shielding and complete concealment of the light source. Ideal for grazing applications. Can be cut to size on site.

Light engine

24V Light source assembled using state of the art automated Reel-to-Reel (R2R) production process supporting LED Linear™ Tj Away® thin flexible circuit board technology.

Equipped with high-quality CSP LEDs with 3-step MacAdam binning (SDCM3), centered on the target CCT (one bin only), and an extended photometric code Wxxx/339 for exceptional color consistency over the entire lifetime. This ensures excellent heat dissipation and a lifetime of >60,000 hours (L80/B10) at 25°C ambient temperature and up to a maximum ambient temperature of 50°C (40°C at 40 W/m).

Excellent color rendering with CRI up to 97 (R9 = 91) and TM-30-15 values up to Rf = 90 / Rg = 101 ensure outstanding light quality. For tunable white, CRI values up to 95 (R9 = 92) and TM-30-15 values up to Rf = 90 / Rg = 100 apply.

Uniform light intensity across the entire length of the luminaire is ensured through active current regulation, controlled by integrated circuits (ICs) at each step of the light source.

Protected against electrostatic discharge up to +/- 2,000 V and polarity miswiring.

Fully PWM dimmable for frequency > 0 Hz up to 2 kHz (flicker free for frequency higher than 1.2 kHz according to IEEE P1789 standard).

Luminaire and light engine engineered and produced in Germany.

Reports and certifications

CE:	Yes, according to EN 60598-1:2022; EN 60598-2-1:2021

Ecodesign Directive 2019/2020/EU (L315/209)

ROHS Directive 2011/65/EU (L174/88)

WEEE Directive 2012/19/EU (L197/38)

Saltwater for coastal regions: 	Yes, according to EN IEC 60068-2-52:2018 testing method: 1

Testing duration 28 days by 40°C with 93% humidity and

5% salt concentration

IK10 impact resistance:		Yes, according to EN 62262:2002 + A1:2021

Stress: 20 J = 5 kg from 40 cm height

(applies only to the version with in-ground installation channel “IF”)

IK08 impact resistance:	Yes, according to EN 62262:2002 + A1:2021
Impact energy: 20 J = 5 kg dropped from a height of 40 cm

Static load for drive-over

and walk-over: 			Yes, according to EN 60598-2-13: 2006 + A11: 2021

IP67:				Yes, testing requirements: 30 min. long, 1 meter under water

ETL:				Yes, according to ATM report

UL2108:2015 Ed.2+R:27Sep2024;

CSA C22.2#250.0:2021 Ed.5+U1;U2

CB-Report: 			Yes, according to IEC 60598-2-1:2020; IEC 60598-1:2020

DarkSky:			Yes, according to DarkSky Association regulations

Light engine up to max. 3000 K

Additional glare control accessory

UV resistance 			Yes, according DIN ISO 4892-2

Testing conditions: doses of UV-A and UV-B radiation equivalent to ten years of direct sunlight exposure

TM30, TM21, TM82:		Reports available on request

LM79, LM80:			Reports available on request