LYRA ECLIPSE IP40 static white

24 V, externally certified, IP40-rated elegant circular luminaire with a slim profile cross-section. Available in two different diameters and two profile colors.

LYRA ECLIPSE: 806 mm (diameter)
LYRA ECLIPSE: 1164 mm (diameter)

Housing made of extruded and curved anodized aluminum profiles in black and silver.

Eclipse is supplied with a matching suspension mounting kit (max. suspension height approx. 2 m) and a ballast for recessed or ceiling mounting. An elegant eye-catcher for outstanding lighting solutions. Includes matching canopy and controller (DALI).

Equipped with a high-quality opal optic.

Light engine

24V Light source assembled using state of the art automated Reel-to-Reel (R2R) production process supporting LED Linear™ Tj Away® thin flexible circuit board technology.

In combination with LED packages, a thermal resistance is achieved that enables optimal heat dissipation (junction-to-profile), resulting in an outstanding lifetime of over 60,000 operating hours (L80/B10).

Integrated high-quality LEDs with 3-step MacAdam binning (SDCM3), centered on a single target color temperature (one bin only) with extended photometric code Wxxx/339, ensure excellent color consistency over the entire rated lifetime.

Outstanding color rendering with Ra values up to 96 (R9 = 81) and TM-30-15 values up to Rf = 89 / Rg = 98 ensures exceptional light quality. Protected against electrostatic discharge up to ±2,000 V.

Uniform light intensity across the entire luminaire is ensured through active current regulation, controlled by integrated circuits (ICs) at each step of the light source.

Consistent light intensity all along the luminaire length is obtained thanks to active current regulation operated by dedicated integrated circuits (ICs) on each step.

Fully PWM-dimmable for frequencies > 0 Hz up to 2 kHz (flicker-free above 1.2 kHz according to IEEE P1789 standard).

Luminaire and light engine engineered and produced in Germany.

Reports and certifications

CE:	Yes, according to EN 60598-1:2022; EN 60598-2-1:2021

Ecodesign Directive 2019/2020/EU (L315/209)

ROHS Directive 2011/65/EU (L174/88)

WEEE Directive 2012/19/EU (L197/38)

ETL:				Yes, according to ATM report

UL2108:2015 Ed.2+R:27Sep2024;

CSA C22.2#250.0:2021 Ed.5+U1;U2

UV resistance 			Yes, according DIN ISO 4892-2

Testing conditions: doses of UV-A and UV-B radiation equivalent to ten years of direct sunlight exposure

TM30, TM21, TM82:		Reports available on request

LM79, LM80:			Reports available on request