LYRA static white, tunable white and RGB

24 V, externally certified, IP40-rated linear luminaire with a stylish, round aluminum profile with a diameter of only 20 mm.

LYRA (linear lens): 13.1 mm x 20 mm (H x W)
LYRA (round cover): 20 mm x 20 mm (H x W)

Housing made of extruded, anodized aluminum profiles in black and silver with screw-fastened aluminum end caps. Equipped with an IP67 cable with open end.

Supports the use of six high-quality optical options: clear, diffuse, or opal round covers for smooth light gradients; linear lenses with a 10° beam angle for grazing lighting; or 30° and 60° for wider light distribution with precise light control.

Mounting accessory

- Suspension mounting with a suspension kit and clip for fixing the luminaire. Suitable for direct ceiling installation. Includes a 2 m suspension wire. Available in black and silver.

- Surface mounting with a mounting clip made of anodized aluminum, including screws for direct installation on wall or ceiling. Available in black and silver.

- Surface mounting with a mounting clip made of black plastic, including screws for direct installation on wall or ceiling. Particularly suitable for vertical installation to prevent slipping and rotation.

Light engine

24V Light source assembled using state of the art automated Reel-to-Reel (R2R) production process supporting LED Linear™ Tj Away® thin flexible circuit board technology.

Integrated high-quality LEDs with 3-step MacAdam binning (SDCM3), centered on a single target color temperature (one bin only) with extended photometric code Wxxx/339, ensure excellent color consistency over the entire rated lifetime.

Static white uses high-quality LEDs that enable optimal heat dissipation (junction-to-profile), achieving an outstanding lifetime of over 60,000 operating hours (L80/B10).
Outstanding color rendering with Ra values up to 98 (R9 = 96) and TM-30-15 values up to Rf = 97 / Rg = 102 ensures remarkable light quality.

Tunable white uses high-quality LEDs that enable optimal heat dissipation (junction-to-profile), achieving an outstanding lifetime of over 60,000 operating hours (L80/B10).
Excellent color rendering with Ra values up to 95 (R9 = 87) and TM-30-15 values up to Rf = 96 / Rg = 104 ensures outstanding light quality.

RGB uses LED packages that enable optimal heat dissipation (junction-to-mounting surface), achieving an outstanding lifetime of 60,000 operating hours (L80/B10).

Protected against electrostatic discharge up to ±2,000 V and reverse polarity.

Consistent light intensity all along the luminaire length is obtained thanks to active current regulation operated by dedicated integrated circuits (ICs) on each step.

Fully PWM dimmable for frequency > 0 Hz up to 2 kHz (flicker free for frequency higher than 1.2 kHz according to IEEE P1789 standard).

Luminaire and light engine engineered and produced in Germany.

Reports and certifications

CE:	Yes, according to EN 60598-1:2022; EN 60598-2-1:2021

Ecodesign Directive 2019/2020/EU (L315/209)

ROHS Directive 2011/65/EU (L174/88)

WEEE Directive 2012/19/EU (L197/38)

ETL:				Yes, according to ATM report

UL2108:2015 Ed.2+R:27Sep2024;

CSA C22.2#250.0:2021 Ed.5+U1;U2

UV resistance 			Yes, according DIN ISO 4892-2

Testing conditions: doses of UV-A and UV-B radiation equivalent to ten years of direct sunlight exposure

TM30, TM21, TM82:		Reports available on request

LM79, LM80:			Reports available on request