MARS ARCHITECTURAL IP40 static white, tunable white and RGBW

24 V, externally certified, IP40-rated slim linear luminaire with integrated glare control.
In a robust aluminum housing.

MARS Type 42 Sym and Opal: 42 mm x 42 mm (H x W)
MARS Type 42 Asym: 47.4 mm x 42 mm (H x W)
MARS Type 62 Sym and Opal: 62 mm x 42 mm (H x W)
MARS Type 62 Asym: 67.4 mm x 42 mm (H x W)

MARS Type 24 Sym and Opal (recessed with frame): 24 mm x 41.8 mm (H x W)
MARS Type 24 Asym (recessed with frame): 29.2 mm x 41.5 mm (H x W)
MARS Type 62 Sym and Opal (recessed with frame): 62 mm x 58 mm (H x W)

Housing made of extruded and anodized aluminum with screw-fastened aluminum end caps, available in white, silver, or black. Supplied with a 2 m open cable.

Optionally available with integrated driver (ON/OFF, DALI, DALI for emergency lighting) and canopy with driver.

Nanometric lens optics enable precise Gaussian light distribution with beam angles of 15°, 25°, 40°, and 65°, without color shift across the beam. Glare control optics are available in black or white, providing a UGR value as low as 7 (depending on the variant; see configuration table) for optimal visual comfort.

Additionally, a wallwash optic with exceptional uniformity and optimal glare control is available (nano optics only available in combination with static white and dynamic white).

An opal PMMA cover provides dot-free and homogeneous light.

Mounting accessory

- Surface mounting by screwing the mounting profile into place.

- Recessed installation with frame and mounting springs for easy installation into drywall ceilings. Mounting springs are included. Available with or without integrated driver.

- Suspension mounting with a 2 m suspension cable for direct ceiling installation. Mounting accessories are included.

All mounting options are available in white, silver, and black.

Light engine

24V Light source assembled using state of the art automated Reel-to-Reel (R2R) production process supporting LED Linear™ Tj Away® thin flexible circuit board technology.

In combination with CSP LEDs (chip-scale package), a thermal resistance of 5.33 K/W is achieved for static and tunable white, enabling optimal heat dissipation (junction-to-profile) and guaranteeing an outstanding lifetime of >102,000 hours (L80/B10). High-quality CSP LEDs with 3-step MacAdam binning (SDCM3), centered on the target CCT (one bin only), with extended photometric code Wxxx/339, ensure exceptional color consistency over the entire rated lifetime.

Premium color rendering with CRI up to 95 (R9 = 88) and TM-30-15 values up to Rf = 91 / Rg = 101 for static white, and CRI up to 88 (R9 = 32) with TM-30-15 values up to Rf = 88 / Rg = 99 for tunable white, ensures excellent light quality.

In combination with ceramic LED packages (RGB) and CSP LEDs (white), RGBW achieves optimal heat dissipation (junction-to-profile), ensuring a lifetime of 60,000 hours (L80/B10 for RGB) and >60,000 hours (L80/B10 for white).

Consistent light intensity all along the luminaire length is obtained thanks to active current regulation operated by dedicated integrated circuits (ICs) on each step.

Fully PWM-dimmable for frequencies > 0 Hz up to 2 kHz (flicker-free above 1.2 kHz according to IEEE P1789 standard).

Luminaire and light engine engineered and produced in Germany.

Reports and certifications

CE:	Yes, according to EN 60598-1:2022; EN 60598-2-1:2021

Ecodesign Directive 2019/2020/EU (L315/209)

ROHS Directive 2011/65/EU (L174/88)

WEEE Directive 2012/19/EU (L197/38)

ETL:				Yes, according to ATM report

UL2108:2015 Ed.2+R:27Sep2024;

CSA C22.2#250.0:2021 Ed.5+U1;U2

UV resistance 			Yes, according DIN ISO 4892-2

Testing conditions: doses of UV-A and UV-B radiation equivalent to ten years of direct sunlight exposure

TM30, TM21, TM82:		Reports available on request

LM79, LM80:			Reports available on request