OCEANOS IP67 static and tunable white and RGB

24 V, IP67-protected and IK10-certified LED linear luminaire made of stainless steel (316L/1.4404/V4A) for in-ground installation in outdoor and indoor applications. Suitable for installation in roadways and parking areas with loads of up to 30 kN.

OCEANOS (total): 100 mm x 125 mm (W x H)
OCEANOS (insert): 34 mm x 37 mm (W x H)

Available in two different lengths: 520 mm and 1020 mm.

Fully specialized hard polyurethane encapsulation ensures high surface hardness and wear resistance, with premium waterproof sealing and UV resistance. The polyurethane formula has been specifically developed for LED technology. Resistant to corrosive gases like  sulfur, chlorine, and salt mist. Fully automated and reliable in-house encapsulation process with 100% functional and outgoing goods inspection of each luminaire.

Equipped with an IP67 cable and connector system with an additional torque coupling for especially secure installation in the ground.

Thanks to its high-quality opal encapsulation, the luminaire provides homogeneous light across the entire emitting surface. Optical and mechanical linking enables continuous lines of light without visible gaps. Grazing applications can be achieved with 10° or 30° optics. The clear encapsulation allows maximum efficiency and targeted light distribution.

Mounting accessory

- Suitable for in-ground installation as a single luminaire. Stainless steel frame in two heights (90 mm and 125 mm). Includes Styrodur dummy for installation in concrete.

- Suitable for in-ground installation as a continuous light line. Stainless steel frame in two heights (90 mm and 125 mm). Recommended to use an end piece/connector at each end of the light line as an anti-vandalism solution. Includes Styrodur dummy for installation in concrete.

- Removal accessories for extracting luminaires from the recessed mounting frame.

Light engine

24V Light source assembled using state of the art automated Reel-to-Reel (R2R) production process supporting LED Linear™ Tj Away® thin flexible circuit board technology.

Static white uses high-quality LEDs with optimal heat dissipation (junction to profile), achieving an outstanding lifetime of over 60,000 operating hours (L80/B10). Integrated high-quality LEDs with 3-step MacAdam binning (SDCM3), centered on a single target color temperature (one bin only). Excellent color rendering with Ra values up to 98 (R9 = 96) and TM-30-15 up to Rf = 97 / Rg = 102, ensuring exceptional light quality. Protection against electrostatic discharge up to +/- 2.000 V and polarity miswiring.

Tunable white uses high-quality LEDs with optimal heat dissipation (junction to profile), achieving an outstanding lifetime of over 60,000 operating hours (L80/B10). Integrated LEDs for dynamic white with 3-step MacAdam binning (SDCM3), centered on a single target color temperature (one bin only) with extended photometric code Wxxx/339, ensuring excellent color consistency over the entire rated lifetime. Premium color rendering with CRI up to 97 (R9 = 96) and TM-30-15 up to Rf = 92 / Rg = 103.

RGB uses LED packages enabling optimal heat dissipation (junction to profile), achieving an outstanding lifetime of 60,000 operating hours (L80/B10). Protection against electrostatic discharge up to ±2,000 V and polarity miswiring.

Consistent light intensity all along the luminaire length is obtained thanks to active current regulation operated by dedicated integrated circuits (ICs) on each step.

Fully PWM dimmable for frequency > 0 Hz up to 2 kHz (flicker free for frequency higher than 1.2 kHz according to IEEE P1789 standard).

Luminaire and light engine engineered and produced in Germany.

Reports and certifications

CE:	Yes, according to EN 60598-1:2022; EN 60598-2-1:2021

Ecodesign Directive 2019/2020/EU (L315/209)

ROHS Directive 2011/65/EU (L174/88)

WEEE Directive 2012/19/EU (L197/38)

Saltwater for coastal regions: 	 Yes, according to EN IEC 60068-2-52:2018 testing method: 1

Testing duration 28 days by 40°C with 93% humidity and

5% salt concentration

IK10 impact resistance:		 Yes, according to EN 62262:2002 + A1:2021

Stress: 20 J = 5 kg from 40 cm height

Static load for drive-over

and walk-over: 			 Yes, according to EN 60598-2-13: 2006 + A11: 2021

IP67:				 Yes, testing requirements: 30 min. long, 1 meter under water

ETL:				 Yes, according to ATM report

UL2108:2015 Ed.2+R:27Sep2024;

CSA C22.2#250.0:2021 Ed.5+U1;U2

UV resistance 	Yes, Testing conditions: doses of UV-A and UV-B radiation equivalent to ten years of direct sunlight exposure

TM30, TM21, TM82:		 Reports available on request

LM79, LM80:			 Reports available on request