ULTIMA IP40 static white and tunable white

24 V, externally certified, IP40-rated ultra-slim linear luminaire with integrated glare control as a single luminaire with exceptionally low space requirements

ULTIMA NANO and OPAL: 10 mm x 13 mm (H x W)
ULTIMA AS: 12.8 mm x 13 mm (H x W)

Housing made of extruded, anodized aluminum profiles in black and silver. Optionally equipped with a cable and connector system (single-sided or double-sided) or an open cable end, matching the luminaire color.

Nanometric lens optics provide precise beam angles (15°, 25°, 40°, 60°) with a Color over Angle (CoA) of 25 K, as well as batwing or asymmetric distributions. ULTIMA is supplied with glare control optics in black or white and a UGR < 13 for optimal visual comfort.

The opal version with a PMMA diffuser enables completely dot-free illumination.

Mounting accessory

- Surface mounting with plastic clips for easy installation. Available in white or black.

- Surface mounting with a continuous high aluminum mounting profile, offering sufficient space to accommodate connectors. Available powder-coated in white (RAL9003) or black (RAL9004). Supplied in luminaire length with end caps included.

- Surface mounting with grid ceiling clips, ideal for installation on T-bars of grid ceilings. Particularly suitable for retrofit installations. Available in white/white and black/white.

- Surface mounting with adjustable brackets allowing alignment of the luminaire from 0° to 130°. Available in black and white.

- Surface and recessed mounting with magnets. Suitable for metal surfaces (e.g. steel) or in combination with adhesive steel strips. For recessed installation, a groove (e.g. dovetail milling in wood) is required.

- Recessed installation with frame, suitable for drywall ceilings or panels. Mounting springs allow easy insertion into ceilings. Available powder-coated in white and black.

- Plaster-in installation. Individual luminaires are mounted with magnets into a mounting profile in drywall ceilings, followed by final plastering.

Light engine
24V Light source assembled using state of the art automated Reel-to-Reel (R2R) production process supporting LED Linear™ Tj Away® thin flexible circuit board technology.

Chip-scale package (CSP) LEDs enable optimal heat dissipation (junction-to-profile), achieving an outstanding lifetime of over 102,000 operating hours (L80/B10). Integrated high-quality CSP LEDs with 3-step MacAdam binning (SDCM3), centered on a single target color temperature (one bin only) with extended photometric code Wxxx/339, ensure excellent color consistency over the entire rated lifetime.

For static white, outstanding color rendering is achieved with Ra values up to 95 (R9 = 88) and TM-30-15 values up to Rf = 91 / Rg = 101, ensuring excellent light quality.
For tunable white, color rendering values reach up to Ra 87 (R9 = 54) and TM-30-15 up to Rf = 87 / Rg = 100, ensuring high light quality.

Consistent light intensity all along the luminaire length is obtained thanks to active current regulation operated by dedicated integrated circuits (ICs) on each step.

Fully PWM-dimmable for frequencies > 0 Hz up to 2 kHz (flicker-free above 1.2 kHz according to IEEE P1789 standard).

Luminaire and light engine engineered and produced in Germany.

Reports and certifications

CE:	Yes, according to EN 60598-1:2022; EN 60598-2-1:2021

Ecodesign Directive 2019/2020/EU (L315/209)

ROHS Directive 2011/65/EU (L174/88)

WEEE Directive 2012/19/EU (L197/38)

ETL:				Yes, according to ATM report

UL2108:2015 Ed.2+R:27Sep2024;

CSA C22.2#250.0:2021 Ed.5+U1;U2

CB-Report: 			Yes, according to IEC 60598-2-1:2020; IEC 60598-1:2020

EPD:				Yes, according to ISO 14025

UV resistance 			Yes, according DIN ISO 4892-2

Testing conditions: doses of UV-A and UV-B radiation equivalent to ten years of direct sunlight exposure

TM30, TM21, TM82:		Reports available on request

LM79, LM80:			Reports available on request