VARIOLED Flex static white and RGB

24 V, externally certified, IP67-protected linear flexible LED light line. Minimal cross-section of 
12-5 mm x 4 mm (W x H), freely configurable in length up to five meters, including a 140 mm cable with IP67 mini connector or open cable, available on one or both sides.

Fully encapsulated in polyurethane with a clear polyurethane top layer, providing high-quality waterproof sealing and UV resistance. The polyurethane formulation has been specifically developed for LED technology. Increased efficiency (higher luminous efficacy) due to the white base. Fully automated and reliable in-house encapsulation process with 100% functional testing and outgoing goods inspection of every luminaire.

An excellent solution for homogeneous indirect outdoor lighting, cove lighting, and other indirect lighting applications with medium distances between illuminated surfaces where IP67 protection is required.

Mounting accessory

- Transparent plastic clips for fixing the luminaire to a surface.

- High-quality double-sided adhesive tape for fixing the luminaire to a surface.

Light engine

24V Light source assembled using state of the art automated Reel-to-Reel (R2R) production process supporting LED Linear™ Tj Away® thin flexible circuit board technology.

Static white uses high-quality LED packages that enable optimal heat dissipation (junction to profile), achieving a lifetime of up to 102,000 operating hours (L80/B10).

Includes LEDs with 3-step MacAdam binning (SDCM3), centered on a single target color temperature (one bin only) with extended photometric code Wxxx/339, ensuring exceptional color consistency over the entire lifetime.

High color rendering performance with Ra values up to 95 (R9 = 82) and TM-30-15 up to Rf = 82 / Rg = 101 for static white, delivering remarkable light quality.

RGB uses LED packages that enable optimal heat dissipation (junction to profile), achieving an outstanding lifetime of 60,000 operating hours (L80/B10). Each step consists of 6 high-quality Japanese “3-in-1” RGB LEDs.

Consistent light intensity all along the luminaire length is obtained thanks to active current regulation operated by dedicated integrated circuits (ICs) on each step.

Protected against electrostatic discharge +/- 2,000 V and polarity miswiring.

Fully PWM dimmable for frequency > 0 Hz up to 2 kHz (flicker free for frequency higher than 1.2 kHz according to IEEE P1789 standard).

Luminaire and light engine engineered and produced in Germany.

Reports and certifications

CE:	Yes, according to EN 60598-1:2022; EN 60598-2-1:2021

Ecodesign Directive 2019/2020/EU (L315/209)

ROHS Directive 2011/65/EU (L174/88)

WEEE Directive 2012/19/EU (L197/38)

Saltwater for coastal regions: 	 Yes, according to EN IEC 60068-2-52:2018 testing method: 1

Testing duration 28 days by 40°C with 93% humidity and

5% salt concentration

IP67:				 Yes, testing requirements: 30 min. long, 1 meter under water

ETL:				 Yes, according to ATM report

UL2108:2015 Ed.2+R:27Sep2024;

CSA C22.2#250.0:2021 Ed.5+U1;U2

UV resistance 	Yes, Testing conditions: doses of UV-A and UV-B radiation equivalent to ten years of direct sunlight exposure

TM30, TM21, TM82:		 Reports available on request

LM79, LM80:			 Reports available on request