VENUS static and tunable white and RGB

24 V light source, manufactured using a state-of-the-art automated reel-to-reel (R2R) production process, supporting LED Linear™ Tj Away® technology for thin, flexible circuit boards.

VENUS TopView (TV): 15 x 16 mm (H x W), bending direction along the vertical axis of the luminaire
VENUS SideView (SV): 20,5 x 10 mm (H x W), bending direction along the horizontal axis of the luminaire

Fully encapsulated in opal polyurethane, providing high-quality waterproof sealing and UV resistance. The polyurethane formulation has been specifically developed for LED technology. Resistant to corrosive gases such as sulfur, chlorine, or salt mist. Fully automated and reliable in-house encapsulation process with 100% functional testing and outgoing goods inspection of each luminaire.

Freely configurable in length up to a maximum of five meters. Equipped with an IP67 cable and connector system, including selectable mounting accessories.

Two variants with different bending directions are available: TopView (TV) with vertical bending and SideView (SV) with horizontal bending. Maximum bending radius of 150 mm. An excellent solution for narrow, organic accent lighting applications (indoor/outdoor), for facades or decorative direct lighting where IP67 protection is required.

Mounting accessory

- Surface mounting with mounting clips, without cable channel, made of powder-coated aluminum in RAL9003 matte. Enables flat surface installation.

- Surface mounting with mounting clips including cable channel, made of powder-coated aluminum in RAL9003 matte. Allows concealed wiring within the mounting clip.

- Surface mounting with mounting profile, without cable channel, made of powder-coated aluminum in RAL9003 matte. Custom colors for profiles available on request. Enables flat surface installation.

- Surface mounting with mounting profile including cable channel, made of powder-coated aluminum in RAL9003 matte. Custom colors for profiles available on request. Allows concealed wiring within the mounting profile.

- Surface mounting with stainless steel clips.

Light engine

24V Light source assembled using state of the art automated Reel-to-Reel (R2R) production process supporting LED Linear™ Tj Away® thin flexible circuit board technology.

The 24 V light source uses high-quality LED packages offering static white, dynamic white, RGB, RGBW, or color options depending on configuration.

Static white uses modern LED packages to ensure optimal heat dissipation (junction to profile), achieving an excellent L80/B10 lifetime of >102,000 hours. Features SDCM3, One Bin Only, and photometric code Wxxx/339, ensuring consistently high color stability over the entire lifetime. Premium color rendering with CRI >92 (R9 >4) and TM-30-15 values up to Rf = 82 / Rg = 97. Protected against electrostatic discharge up to +/- 8.000 V and polarity miswiring.

Tunable white is based on LED packages ideal for applications requiring precise tuning of color temperatures along the Planckian curve. The LEDs feature 3-step MacAdam binning (SDCM3), One Bin Only, and extended photometric code Wxxx/339, ensuring excellent color consistency over the entire lifetime. Premium color rendering with CRI 95 (R9 = 77) and TM-30-15 values up to Rf = 92 / Rg = 103.

RGB uses high-quality RGB LEDs for optimal heat dissipation (junction to profile) and a lifetime of >60,000 operating hours (L80/B10). Active current regulation ensures uniform light intensity over the entire strip length. Protected against electrostatic discharge up to ±2,000 V and polarity miswiring.

Fully PWM dimmable for frequency > 0 Hz up to 2 kHz (flicker free for frequency higher than 1.2 kHz according to IEEE P1789 standard).

Luminaire and light engine engineered and produced in Germany.

Reports and certifications

CE:	Yes, according to EN 60598-1:2022; EN 60598-2-1:2021

Ecodesign Directive 2019/2020/EU (L315/209)

ROHS Directive 2011/65/EU (L174/88)

WEEE Directive 2012/19/EU (L197/38)

Saltwater for coastal regions: 	Yes, according to EN IEC 60068-2-52:2018 testing method: 1

Testing duration 28 days by 40°C with 93% humidity and

5% salt concentration

IP67:				Yes, testing requirements: 30 min. long, 1 meter under water

ETL:				Yes, according to ATM report

UL2108:2015 Ed.2+R:27Sep2024;

CSA C22.2#250.0:2021 Ed.5+U1;U2

CB-Report: 			Yes, according to IEC 60598-2-1:2020; IEC 60598-1:2020

EPD:				Yes, according to ISO14025

UV resistance 			Yes, according DIN ISO 4892-2

Testing conditions: doses of UV-A and UV-B radiation equivalent to ten years of direct sunlight exposure

TM30, TM21, TM82:		Reports available on request

LM79, LM80:			Reports available on request