VENUS TC static white

24 V, externally certified, IP67-rated flexible and completely homogeneous, dot-free LED light line.

VENUS TC TopView and 3D: 18 x 18 mm (H x W)

The form factor does not differ between the TopView and 3D variants, allowing a continuous light line combining both types. Both variants can be configured either as an opal light line or with an integrated flexible optic providing a narrow beam distribution.

Fully encapsulated in polyurethane, providing high-quality waterproof sealing and UV resistance. Opal polyurethane is used for TopView variants, while clear polyurethane is used for versions with flexible linear optics. The polyurethane formulation has been specifically developed for LED technology. Resistant to corrosive gases such as sulfur, chlorine, or salt mist. Fully automated and reliable in-house encapsulation process with 100% functional testing and outgoing goods inspection of each luminaire.

Freely configurable in length up to a maximum of four meters for the TV variant and two meters for the 3D variant. Rear-side cable routing enables optimal installation as a continuous light line. Equipped with an IP67 cable and connector system, including selectable mounting accessories.

True Color technology ensures optimal reproduction of the LED spectrum (minimal color shift). Available as 2D (Top View) or 3D. The 3D version is bendable in three dimensions with a minimum radius of 150 mm and can be twisted up to 180°/m. The 2D version is bendable only in the vertical direction, with a radius of 150 mm – an ideal solution for narrow, organic accent lighting applications (indoor/outdoor), facades, or decorative direct lighting requiring IP67 protection.

Two light distribution options are available: an elliptical 20° beam or an opal optic. As a unique linear optic technology, the 20° optic provides homogeneous and precise light distribution without unwanted scalloping effects (as seen with TIR lenses).

Mounting accessory

- Surface mounting with mounting clips, without cable channel, made of powder-coated aluminum in RAL9003 matte. Enables flat surface installation.

- Surface mounting with mounting clips including cable channel, made of powder-coated aluminum in RAL9003 matte. Allows concealed wiring within the mounting clip.

- Surface mounting with mounting profile, without cable channel, made of powder-coated aluminum in RAL9003 matte. Custom colors available on request. Enables flat surface installation.

- Surface mounting with mounting profile including cable channel, made of powder-coated aluminum in RAL9003 matte. Custom colors available on request. Allows concealed wiring within the mounting profile.

- Surface mounting with retaining clips made of transparent plastic.

Additional glare control accessory

- Aluminum glare shield profile, powder-coated in RAL9003 matte, for shielding light to one side and completely concealing the light source. Ideal for grazing applications. Available in 1 m and 2 m lengths. Suitable for use with mounting clips and mounting profiles.

Light engine

24V Light source assembled using state of the art automated Reel-to-Reel (R2R) production process supporting LED Linear™ Tj Away® thin flexible circuit board technology.

In combination with chip-scale package (CSP) LEDs featuring 3-step MacAdam binning (SDCM3), centered on the target CCT (one bin only) and extended photometric code Wxxx/339, exceptional color consistency over the rated lifetime is ensured. Reduced thermal resistance enables optimal heat dissipation (junction to profile), guaranteeing an excellent lifetime of >60,000 hours (L80/B10).

Premium color rendering with CRI up to 96 (R9 up to 90) and TM-30-15 values up to Rf = 92 / Rg = 100 ensures outstanding light quality.

Uniform light intensity across the entire length of the luminaire is achieved through active current regulation via integrated circuits (ICs) at each step of the light source.

Protected against electrostatic discharge up to ±2,000 V and polarity miswiring

Fully PWM dimmable for frequency > 0 Hz up to 2 kHz (flicker free for frequency higher than 1.2 kHz according to IEEE P1789 standard).

Luminaire and light engine engineered and produced in Germany.

Reports and certifications

CE:	Yes, according to EN 60598-1:2022; EN 60598-2-1:2021

Ecodesign Directive 2019/2020/EU (L315/209)

ROHS Directive 2011/65/EU (L174/88)

WEEE Directive 2012/19/EU (L197/38)

Saltwater for coastal regions: 	Yes, according to EN IEC 60068-2-52:2018 testing method: 1

Testing duration 28 days by 40°C with 93% humidity and

5% salt concentration

IP67:				Yes, testing requirements: 30 min. long, 1 meter under water

ETL:				Yes, according to ATM report

UL2108:2015 Ed.2+R:27Sep2024;

CSA C22.2#250.0:2021 Ed.5+U1;U2

CB-Report: 			Yes, according to IEC 60598-2-1:2020; IEC 60598-1:2020

EPD:				Yes, according to ISO14025

DarkSky:			Yes, according to DarkSky Association regulations

Light engine up to max. 3000 K

Integrated glare control

Additional glare control accessory

(applies only to the version with optics)

UV resistance 			Yes, according DIN ISO 4892-2

Testing conditions: doses of UV-A and UV-B radiation equivalent to ten years of direct sunlight exposure

TM30, TM21, TM82:		Reports available on request

LM79, LM80:			Reports available on request