XOOCOVE IP40 static white

24 V, externally certified, IP40-rated linear luminaire, specifically designed for cove lighting, featuring tool-free, fast, and convenient installation.

XOOCOVE (round cover): 20 mm x 20 mm (H x W)
XOOCOVE (linear lens): 13.1 mm x 20 mm (H x W)

Available in six standard lengths ranging from 260 mm to 2510 mm.

Housing made of extruded, anodized aluminum profiles in black and silver. Equipped with an IP20 cable and connector system. Each luminaire is connected to the bus line (24 V, max. 8 A, 190 W) using special piercing connectors. With rear cable feed. Includes selected mounting accessories.

Supports the use of two high-quality optics: a diffuse round cover for smooth light gradients and a linear lens with a 60° beam angle for wide light distribution with precise light control.

Mounting accessory

- Tool-free installation with silicone mounting pads. Allows adjustment of the luminaire alignment. Cable fixation is integrated into grooves within the pad. Requires a cove width of at least 55 mm. Color: white.

- Surface mounting with plastic mounting clips. Suitable for horizontal and vertical surfaces where silicone pads cannot be used. Allows free adjustment of the luminaire alignment. 
Color: black.

Light engine

24V Light source assembled using state of the art automated Reel-to-Reel (R2R) production process supporting LED Linear™ Tj Away® thin flexible circuit board technology.

In combination with high-quality LED packages, a thermal resistance is achieved that enables optimal heat dissipation (junction-to-profile), resulting in an outstanding lifetime of over 102,000 operating hours (L80/B10). Integrated high-quality LEDs with 3-step MacAdam binning (SDCM3), centered on a single target color temperature (one bin only) with extended photometric code Wxxx/339, ensure excellent color consistency over the entire rated lifetime.

Outstanding color rendering with Ra values up to 97 (R9 = 91) and TM-30-15 values up to Rf = 90 / Rg = 101.

Protected against electrostatic discharge up to ±4,000 V and reverse polarity.

Consistent light intensity all along the luminaire length is obtained thanks to active current regulation operated by dedicated integrated circuits (ICs) on each step.

Fully PWM-dimmable for frequencies > 0 Hz up to 2 kHz (flicker-free above 1.2 kHz according to IEEE P1789 standard).

Luminaire and light engine engineered and produced in Germany.

Reports and certifications

CE:	Yes, according to EN 60598-1:2022; EN 60598-2-1:2021

Ecodesign Directive 2019/2020/EU (L315/209)

ROHS Directive 2011/65/EU (L174/88)

WEEE Directive 2012/19/EU (L197/38)

ETL:				Yes, according to ATM report

UL2108:2015 Ed.2+R:27Sep2024;

CSA C22.2#250.0:2021 Ed.5+U1;U2

UV resistance 			Yes, according DIN ISO 4892-2

Testing conditions: doses of UV-A and UV-B radiation equivalent to ten years of direct sunlight exposure

TM30, TM21, TM82:		Reports available on request

LM79, LM80:			Reports available on request