XOOLIGHT IP67 static white, tunable white and RGB

24 V, externally certified, IP67-rated linear luminaire for general lighting.

XOOLIGHT (surface mounted): 		62 mm x 42 mm (H x W)
XOOLIGHT (recessed with trim): 	62 mm x 58 mm (H x W)

Freely scalable up to a length of 4 m.

Fully encapsulated in polyurethane with a clear polyurethane top layer, providing high-quality waterproof sealing and UV resistance. The polyurethane formulation has been specifically developed for LED technology. Resistant to corrosive gases such as sulfur, chlorine, or salt mist. Fully automated and reliable in-house encapsulation process with 100% functional testing and outgoing goods inspection of each luminaire.

Housing made of extruded aluminum profile in silver anodized E6/C0 or white powder-coated RAL9003. Supplied with a 2 m black cable with open end. Optionally available with end or rear cable feed. Includes mounting accessories as selected.

Mounting accessory

- Pendant mounting with a suspension kit for direct ceiling installation. The suspension wire can be shortened on site.

- Recessed installation using mounting clips. The height of the clips can be adapted to the ceiling thickness before clipping in the lighting insert.

- Surface mounting without additional accessories using a specially designed mounting profile. Holes must be drilled on site.

Light engine

24V Light source assembled using state of the art automated Reel-to-Reel (R2R) production process supporting LED Linear™ Tj Away® thin flexible circuit board technology.

In combination with LED packages, a thermal resistance is achieved that enables optimal heat dissipation (junction-to-profile), resulting in an outstanding lifetime of over 60,000 operating hours (L80/B10). Integrated high-quality LEDs with 3-step MacAdam binning (SDCM3), centered on a single target color temperature (one bin only) with extended photometric code Wxxx/339, ensure excellent color consistency throughout the entire rated lifetime.

For static white, outstanding color rendering is achieved with Ra values up to 95 (R9 = 82) and TM-30-15 values up to Rf = 90 / Rg = 101. For tunable white, premium color rendering is achieved with CRI 95 (R9 = 77) and TM-30-15 values up to Rf = 92 / Rg = 103.

Protection against electrostatic discharge up to ±2,000 V and polarity miswiring.

Consistent light intensity all along the luminaire length is obtained thanks to active current regulation operated by dedicated integrated circuits (ICs) on each step.

Fully PWM dimmable for frequency > 0 Hz up to 2 kHz (flicker free for frequency higher than 1.2 kHz according to IEEE P1789 standard).

Luminaire and light engine engineered and produced in Germany.

Reports and certifications

CE:	Yes, according to EN 60598-1:2022; EN 60598-2-1:2021

Ecodesign Directive 2019/2020/EU (L315/209)

ROHS Directive 2011/65/EU (L174/88)

WEEE Directive 2012/19/EU (L197/38)

IP67:				Yes, testing requirements: 30 min. long, 1 meter under water

ETL:				Yes, according to ATM report

UL2108:2015 Ed.2+R:27Sep2024;

CSA C22.2#250.0:2021 Ed.5+U1;U2

UV restistance 			Yes, according DIN ISO 4892-2

Testing conditions: doses of UV-A and UV-B radiation equivalent to ten years of direct sunlight exposure

TM30, TM21, TM82:		Reports available on request

LM79, LM80:			Reports available on request