XOOLINE IP40 static white, tunable white, RGB and RGBW

24 V, externally certified, IP40-rated linear luminaire with outstanding modularity and particularly easy installation.

XOOLINE linear lens: 		19.5 mm x 19.9 mm (W x H)
XOOLINE round cover: 		19.5 mm x 26.9 mm (W x H)
XOOLINE flat cover:		19.5 mm x 19.8 mm (W x H)
XOOLINE high cover: 		19.5 mm x 26.9 mm (W x H)

Housing made of extruded, anodized aluminum profiles in black E6/C35 and silver E6/C0. Equipped with an IP67 cable and connector system. Optionally available with side, end, or rear cable feed. Includes mounting accessories as selected. Diffuse/semi-transparent, screw-fastened end caps ensure an optimal continuous line installation without dark spots.

Enables the use of eight high-quality optical options: clear, diffuse, or opal covers for smooth light gradients, as well as linear lenses with 10°, 30°, or 60° beam angles for precise light control.

Mounting accessory

- Surface mounting with continuous mounting profile made of anodized aluminum (black E6/C35 and silver E6/C0), ensuring visually uniform installation of the linear luminaire.

- Surface mounting with mounting clips made of anodized aluminum (silver), available with four tilt options: 0°, 15°, 30°, and 45°.

- Wall mounting with a continuous mounting profile made of anodized aluminum (silver), ensuring secure installation with light direction parallel to the wall.

- Wall mounting with an adjustable arm made of powder-coated aluminum (RAL9022 – grey or RAL9004 – black) with integrated cable channel. Four wall distances available: 40 mm, 100 mm, 200 mm, and 300 mm; custom lengths available on request. The luminaire can be continuously tilted by ±120°.

- Surface mounting with a 0° plastic mounting clip available in white.

- Surface mounting with an adjustable stainless steel mounting clip, allowing luminaire angle adjustment up to 180°. Only available in combination with the side cable version.

Light engine

24V Light source assembled using state of the art automated Reel-to-Reel (R2R) production process supporting LED Linear™ Tj Away® thin flexible circuit board technology.

Static white uses ceramic LED packages for optimal heat dissipation (junction-to-profile), achieving an outstanding lifetime of over 60,000 operating hours (L80/B10). Integrated high-quality Japanese LEDs with 3-step MacAdam binning (SDCM3), centered on a single target color temperature (one bin only) with extended photometric code Wxxx/339, ensure exceptional color consistency over the entire lifetime. Excellent color rendering with Ra values up to 96 (R9 = 81) and TM-30-15 values up to Rf = 89 / Rg = 98. Protection against electrostatic discharge up to +/- 2,000 V and polarity miswiring.

Tunable white uses high-quality 2016 ceramic packages with 3-step MacAdam binning (SDCM3), centered on a single target color temperature (one bin only) and extended photometric code Wxxx/339, ensuring excellent color consistency over the entire lifetime. Premium color rendering with CRI up to 95 (R9 = 76) and TM-30-15 values up to Rf = 91 / Rg = 103.

RGB uses ceramic LED packages with a thermal resistance of 45.5 K/W, enabling optimal heat dissipation (junction-to-mounting surface) and achieving an outstanding lifetime of 60,000 hours (L80/B10). Integrated high-quality Japanese “3-in-1” RGB LEDs.

RGBW uses ceramic LED packages and chip-scale package (CSP) LEDs in white, achieving a thermal resistance of 43.3 K/W for optimal heat dissipation (junction-to-mounting surface). Outstanding lifetime of 30,000 operating hours (L80/B10 for RGB) and over 60,000 hours (L80/B10 for white). Protection against electrostatic discharge up to +/- 2,000 V.

Consistent light intensity all along the luminaire length is obtained thanks to active current regulation operated by dedicated integrated circuits (ICs) on each step.

Fully PWM dimmable for frequency > 0 Hz up to 2 kHz (flicker free for frequency higher than 1.2 kHz according to IEEE P1789 standard).

Luminaire and light engine engineered and produced in Germany.

Reports and certifications

CE:	Yes, according to EN 60598-1:2022; EN 60598-2-1:2021

Ecodesign Directive 2019/2020/EU (L315/209)

ROHS Directive 2011/65/EU (L174/88)

WEEE Directive 2012/19/EU (L197/38)

ETL:				Yes, according to ATM report

UL2108:2015 Ed.2+R:27Sep2024;

CSA C22.2#250.0:2021 Ed.5+U1;U2

CB-Report: 			Yes, according to IEC 60598-2-1:2020; IEC 60598-1:2020

EPD:				Yes, according to ISO14025

UV resistance 			Yes, according DIN ISO 4892-2

Testing conditions: doses of UV-A and UV-B radiation equivalent to ten years of direct sunlight exposure

TM30, TM21, TM82:		Reports available on request

LM79, LM80:			Reports available on request