XOOLINE IP67 static white, tunable white and RGB

24 V, externally certified, IP67-rated linear luminaire with outstanding modularity and particularly easy installation

XOOLINE round cover: 19.5 mm x 26.9 mm (W x H)
XOOLINE flat cover: 19.5 mm x 19.8 mm (W x H)
XOOLINE high cover: 19.5 mm x 26.9 mm (W x H)

Fully encapsulated in polyurethane with a clear polyurethane top layer, providing high-quality waterproof sealing and UV resistance. The polyurethane formulation has been specifically developed for LED technology. Resistant to corrosive gases such as sulfur, chlorine, or salt mist. Fully automated and reliable in-house encapsulation process with 100% functional testing and outgoing goods inspection of each luminaire.

Housing made of extruded, anodized aluminum profiles in black E6/C35 and silver E6/C0. Equipped with an IP67 cable and connector system. Optionally available with side, end, or rear cable feed. Includes mounting accessories as selected. Diffuse/semi-transparent, screw-fastened end caps ensure an optimal continuous line installation without dark spots.

Diffuse or opal optics for smooth light gradients.

Mounting accessory

- Surface mounting with a continuous mounting profile made of anodized aluminum (black E6/C35 and silver E6/C0), ensuring a visually uniform installation of the linear luminaire.

- Surface mounting with mounting clips made of anodized aluminum (silver), available with four tilt positions: 0°, 15°, 30°, and 45°.

- Wall mounting with a continuous mounting profile made of anodized aluminum, ensuring secure installation with light direction parallel to the wall.

- Wall mounting with an adjustable arm made of powder-coated aluminum (RAL9022 – grey or RAL9004 – black) with integrated cable channel. Four wall offsets are available: 40 mm, 100 mm, 200 mm, and 300 mm; custom lengths available on request. The luminaire can be continuously adjusted by ±120°.

- Surface mounting with a plastic mounting clip available in white.

Light engine

24V Light source assembled using state of the art automated Reel-to-Reel (R2R) production process supporting LED Linear™ Tj Away® thin flexible circuit board technology. 

Integrated high-quality LEDs with 3-step MacAdam binning (SDCM3), centered on a single target color temperature (one bin only) with extended photometric code Wxxx/339, ensure exceptional color consistency over the entire lifetime.

Static white uses ceramic LED packages for optimal heat dissipation (junction-to-profile), achieving an outstanding lifetime of over 60,000 operating hours (L80/B10). Excellent color rendering with Ra values up to 95 (R9 = 74) and TM-30-15 values up to Rf = 91 / Rg = 102. Protection against electrostatic discharge up to ±2,000 V and reverse polarity.

Tunable white uses high-quality 2016 packages. Premium color rendering with CRI 95 (R9 = 77) and TM-30-15 values up to Rf = 91 / Rg = 103.

RGB uses ceramic LED packages that enable optimal heat dissipation (junction-to-mounting surface), achieving an outstanding lifetime of 60,000 hours (L80/B10). Integrated high-quality Japanese “3-in-1” RGB LEDs.

Consistent light intensity all along the luminaire length is obtained thanks to active current regulation operated by dedicated integrated circuits (ICs) on each step.

Fully PWM dimmable for frequency > 0 Hz up to 2 kHz (flicker free for frequency higher than 1.2 kHz according to IEEE P1789 standard).

Luminaire and light engine engineered and produced in Germany.

Reports and certifications

CE:	Yes, according to EN 60598-1:2022; EN 60598-2-1:2021

Ecodesign Directive 2019/2020/EU (L315/209)

ROHS Directive 2011/65/EU (L174/88)

WEEE Directive 2012/19/EU (L197/38)

IP67:				Yes, testing requirements: 30 min. long, 1 meter under water

ETL:				Yes, according to ATM report

UL2108:2015 Ed.2+R:27Sep2024;

CSA C22.2#250.0:2021 Ed.5+U1;U2

UV resistance 			Yes, according DIN ISO 4892-2

Testing conditions: doses of UV-A and UV-B radiation equivalent to ten years of direct sunlight exposure

TM30, TM21, TM82:		Reports available on request

LM79, LM80:			Reports available on request