XOOLUM IP67 static white, tunable white and RGB

24 V, externally certified, IP67-rated linear luminaire with outstanding modularity, compact design, and adjustable luminaire head (adjustable +/- 45°).

XOOLUM (not tilted): 25.4 mm x 31 mm (W x H)
XOOLUM (tilted 45°): 25.4 mm x 36 mm (W x H)

Fully encapsulated in polyurethane with a clear or opal polyurethane top layer, providing high-quality waterproof sealing and UV resistance. The polyurethane formulation has been specifically developed for LED technology. Resistant to corrosive gases such as sulfur, chlorine, or salt mist. Fully automated and reliable in-house encapsulation process with 100% functional testing and outgoing goods inspection of each luminaire.

Housing made of extruded, anodized aluminum profiles in black E6/C35 and silver E6/C0. Equipped with an IP67 cable and connector system. Available with end or rear cable feed. Includes selected mounting accessories.

Optionally available with a highly efficient wall-washer reflector for homogeneous wall illumination, a precise 65° reflector for uniform general/task lighting, or an opal polyurethane top layer.

Mounting accessory

- Surface mounting with a screw-fixed continuous aluminum mounting profile (profile is part of the luminaire). Ensures secure installation and allows ±45° adjustment of the luminaire head. Available in silver anodized and black anodized.

- Surface mounting with an adjustable stainless steel clip, allowing an additional 45° tilt of the luminaire (in addition to the ±45° adjustment of the luminaire head).

- Suspension mounting with a suspension kit and adjustable cable suspension on the luminaire side, for direct ceiling installation. Available in silver and black.

Light engine

24V Light source assembled using state of the art automated Reel-to-Reel (R2R) production process supporting LED Linear™ Tj Away® thin flexible circuit board technology.

Static white and tunable white use ceramic LED packages, achieving low thermal resistance for optimal heat dissipation (junction-to-profile) and resulting in an outstanding lifetime of over 60,000 operating hours (L80/B10). Integrated high-quality LEDs with 3-step MacAdam binning (SDCM3), centered on a single target color temperature (one bin only) with extended photometric code Wxxx/339, ensure excellent color consistency over the entire rated lifetime.

Outstanding color rendering for static white with Ra values up to 95 (R9 = 82) and TM-30-15 values up to Rf = 90 / Rg = 101.
For tunable white with opal optic variants, excellent color rendering is achieved with Ra values up to 95 (R9 = 77) and TM-30-15 values up to Rf = 92 / Rg = 103.

Protected against electrostatic discharge up to +/- 2,000 V and polarity miswiring.

RGB uses high-quality LED packages enabling optimal heat dissipation (junction-to-mounting surface), achieving an outstanding lifetime of over 60,000 operating hours (L80/B10).

Consistent light intensity all along the luminaire length is obtained thanks to active current regulation operated by dedicated integrated circuits (ICs) on each step.

Fully PWM dimmable for frequency > 0 Hz up to 2 kHz (flicker free for frequency higher than 1.2 kHz according to IEEE P1789 standard).

Luminaire and light engine engineered and manufactured in Germany.

Reports and certifications

CE:	Yes, according to EN 60598-1:2022; EN 60598-2-1:2021

Ecodesign Directive 2019/2020/EU (L315/209)

ROHS Directive 2011/65/EU (L174/88)

WEEE Directive 2012/19/EU (L197/38)

Saltwater for coastal regions: 	Yes, according to EN IEC 60068-2-52:2018 testing method: 1

Testing duration 28 days by 40°C with 93% humidity and

5% salt concentration

IP67:				Yes, testing requirements: 30 min. long, 1 meter under water

ETL:				Yes, according to ATM report

UL2108:2015 Ed.2+R:27Sep2024;

CSA C22.2#250.0:2021 Ed.5+U1;U2

UV restistance 			Yes, according DIN ISO 4892-2

Testing conditions: doses of UV-A and UV-B radiation equivalent to ten years of direct sunlight exposure

TM30, TM21, TM82:		Reports available on request

LM79, LM80:			Reports available on request