XOOTUBE static white, tunable white and RGB

24 V, externally certified, IP40-rated linear luminaire with a slim, round design combining industrial style with elegant aesthetics

XOOTUBE suspended: 40 mm x 40 mm (H x W)
XOOTUBE surface-mounted: 80 mm x 42 mm (H x W)

Can be configured as a horizontal or vertical pendant luminaire or as a surface-mounted luminaire (ceiling or wall, each horizontal or vertical). The mounting profile is available in three colors. Elegant aluminum end caps are available in black, white, and silver.

The opal tube enables a 300° light distribution.

Mounting accessory

- Surface mounting with an aluminum profile available in three colors: black, white, and silver. The mounting profile is generously dimensioned to accommodate an optional low-profile driver and/or controller. Suitable for both wall and ceiling mounting.

- Suspension mounting with 4 m suspension cables. All mounting components are included. Horizontal pendant luminaires with a length of 1.3 m or more are equipped with an additional suspension point.

Light engine

24V Light source assembled using state of the art automated Reel-to-Reel (R2R) production process supporting LED Linear™ Tj Away® thin flexible circuit board technology.

Integrated high-quality LEDs with 3-step MacAdam binning (SDCM3), centered on a single target color temperature (one bin only) with extended photometric code Wxxx/339, ensure exceptional color consistency over the entire lifetime.

Static white uses high-quality LEDs enabling optimal heat dissipation (junction-to-profile), achieving an outstanding lifetime of over 102,000 operating hours (L90/B10). Outstanding color rendering with CRI up to 98 (R9 = 96) and TM-30-15 values up to Rf = 96 / Rg = 102.

Tunable white uses high-quality LEDs enabling optimal heat dissipation (junction-to-profile), achieving an outstanding lifetime of over 60,000 operating hours (L80/B10). Excellent color rendering with CRI up to 95 (R9 = 77) and TM-30-15 values up to Rf = 92 / Rg = 103.

RGB uses high-quality LEDs enabling optimal heat dissipation (junction-to-profile), ensuring an outstanding lifetime of 60,000 hours (L80/B10).

Consistent light intensity all along the luminaire length is obtained thanks to active current regulation operated by dedicated integrated circuits (ICs) on each step.

Fully PWM-dimmable for frequencies > 0 Hz up to 2 kHz (flicker-free above 1.2 kHz according to IEEE P1789 standard).

Luminaire and light engine engineered and produced in Germany.

Reports and certifications

CE:	Yes, according to EN 60598-1:2022; EN 60598-2-1:2021

Ecodesign Directive 2019/2020/EU (L315/209)

ROHS Directive 2011/65/EU (L174/88)

WEEE Directive 2012/19/EU (L197/38)

Stress: 20 J = 5 kg from 40 cm height

ETL:				Yes, according to ATM report

UL2108:2015 Ed.2+R:27Sep2024;

CSA C22.2#250.0:2021 Ed.5+U1;U2

UV resistance 			Yes, according DIN ISO 4892-2

Testing conditions: doses of UV-A and UV-B radiation equivalent to ten years of direct sunlight exposure

TM30, TM21, TM82:		Reports available on request

LM79, LM80:			Reports available on request